SUBJECT:			Derating Curve for 34044-3

author:			larry lim

TECHNICIAN:		larry lim

REPORT NO:		ENG-11-SEPT-2020-2

date:			09/11/2020

- Part Description:

34044-4-000 Rev G

SW: 0112

- Equipment List:

High current test fixture. 80TK Thermocouple Module, Multimeter, Espec Test Chamber

- Test Setup:

|  | Unit is first run at ambient. Total temperature increased is subtracted from 80°C to be used as a starting point for derating curve. Next point on curve would be to start with a minimum load and slowly increase the load until temperature reaches 80°C |
| --- | --- |

- Results:

| Ambient: 27.2°C Load Current: 52.6A Highest Temperature Recorded: 50.8°C Total Temperature Increase:23.6 °C 56.4°C starting point for derating curve. |  |
| --- | --- |
| Set temperature: 65°C Load Current: 45.5A Highest Temperature Recorded: 78.6°C Total Temperature Increase: 13.6°C |  |

- Reference Data: