SUBJECT:			Derating Curve for 34044-5

author:			larry lim

TECHNICIAN:		larry lim

REPORT NO:		ENG-11-SEPT-2020-1

date:			09/11/2020

- Part Description:

34044-5-000 Rev C

SW: 0115

- Equipment List:

High current test fixture. 80TK Thermocouple Module, Multimeter, Espec test chamber

- Test Setup:

|  | Unit is first run at ambient. Total temperature increased is subtracted from 80°C to be used as a starting point for derating curve. Next point on curve would be to start with a minimum load and slowly increase the load until temperature reaches 80°C |
| --- | --- |

- Results:

| Ambient: 26.7°C Load Current: 56.2 A Highest Temperature Recorded: 55.4°C Total Temperature Increase: 28.7°C 51.3°C starting point for derating curve. |  |
| --- | --- |
| Set temperature: 65°C Load Current: 40.3 A Highest Temperature Recorded: 78.4°C Total Temperature Increase: 10.4°C |  |

- Reference Data: